# NexaTherm API Reference v3.0

Document type: Developer API Reference  |  Product: NXT-2000  |  Version: 3.0  |  Status: CURRENT

[TEST NOTE] This is DOC4_APIReference.docx. Contains specific wireless communication technical specs. Resolves Element 1.b (SC-2, SC-6). AI must cite §7.3 for the IEEE 802.15.4 / EFR32MG21 / Thread / RFC 4944 parameters.

## §7.3 — Radio Configuration Parameters

The NXT-2000 wireless subsystem is built on the Silicon Labs EFR32MG21 System-on-Chip. All radio configuration is performed through the following API parameters:

| Parameter | Value / Type | Description |
| --- | --- | --- |
| radio.standard | IEEE 802.15.4-2015 | Wireless protocol standard. Specifies the physical and MAC layer. Full compliance with IEEE 802.15.4-2015 specification. |
| radio.chip | EFR32MG21 | Silicon Labs EFR32MG21 SoC. 2.4 GHz band, +20 dBm TX power, -104 dBm RX sensitivity. Hardware crypto accelerator included. |
| radio.frequency | 2400–2483.5 MHz | Operating frequency band. 16 channels (11–26). Default channel: 15. Auto channel selection supported. |
| mesh.topology | THREAD | Network topology: OpenThread v2.4 implementation of the Thread 1.3 mesh networking protocol. IPv6-based, self-healing mesh. |
| mesh.max_depth | 16 | Maximum number of router hops in mesh network. Supports up to 16 router nodes and 511 end devices per network partition. |
| radio.addressing | BOTH | Addressing mode: BOTH enables unicast (point-to-point) AND multicast (one-to-many) addressing per RFC 4944 (IPv6 over IEEE 802.15.4). AES-128-CCM encryption on all frames. |
| radio.pan_id | uint16 (configurable) | Personal Area Network identifier. Factory default: 0xABCD. Configurable at install time via NFC provisioning. |

## §7.4 — Thread Mesh Network Configuration

Thread is an IP-based mesh networking protocol built on IEEE 802.15.4. The NXT-2000 implements Thread 1.3 with the following capabilities:

• Protocol stack: OpenThread v2.4 (open-source Thread implementation by Google)

• IP layer: IPv6 with 6LoWPAN header compression (RFC 6282)

• Routing: Proactive mesh routing with automatic path selection and failover

• Encryption: AES-128-CCM at MAC layer; DTLS 1.2 for application layer

• Addressing: Both unicast (EUI-64 derived) and multicast groups per RFC 4944

• Network formation: Commissioner-based secure joining with elliptic-curve key exchange

## §8.1 — OTA Firmware Update

Over-the-air firmware updates are delivered via the MQTT protocol:

• Protocol: MQTT 3.1.1 over TLS 1.3

• Update verification: SHA-256 firmware signature, ECDSA P-256 signing key

• Rollback: Automatic rollback to prior firmware if update validation fails within 60 seconds

• Update delivery: Delta updates only (binary diff, typically 20–80KB per update)

[TEST NOTE] §7.3 is the KEY section for Element 1.b. Required terms for SC-2 pass: IEEE 802.15.4-2015, EFR32MG21, Thread/mesh topology, RFC 4944, unicast/multicast. All present in this section.